Performance Specification

| Model |  |  |  |  |  | Maximum Time |  | Resistance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vmax | Imax | Ihold | Itrip | Pd | To Trip |  |  |  |  |
|  |  |  |  |  | Typ. | Current | Time | Rimin | Rimax | R1max |
|  | (Vdc) | (A) | (A) | (A) | (W) | (A) | (Sec) | () | () | () |
| 60v0.05A | 60 | 40 | 0.05 | 0.1 | 0.3 | 0.25 | 5 | 7.5 | 20 | 30 |
| 60v0.1A | 60 | 40 | 0.1 | 0.2 | 0.38 | 0.50 | 5 | 2.5 | 7.5 | 12 |
| 60v0.17A | 60 | 40 | 0.17 | 0.34 | 0.48 | 0.85 | 5 | 2.84 | 5.21 | 8 |
| 60V0.2A | 60 | 40 | 0.2 | 0.4 | 0.41 | 1 | 5 | 1.83 | 2.84 | 4.5 |
| 60V0.25A | 60 | 40 | 0.25 | 0.5 | 0.45 | 1.25 | 5 | 1.25 | 1.95 | 3 |
| 60V0.3A | 60 | 40 | 0.3 | 0.6 | 0.49 | 1.5 | 5 | 0.88 | 1.38 | 2.2 |
| 60V0.4A | 60 | 40 | 0.4 | 0.8 | 0.56 | 2 | 5 | 0.55 | 0.88 | 1.4 |
| 60V0.5A | 60 | 40 | 0.5 | 1.0 | 0.77 | 2.5 | 5 | 0.50 | 0.79 | 1.2 |
| 60V0.65A | 60 | 40 | 0.6 | 1.2 | 0.88 | 3.25 | 5 | 0.31 | 0.5 | 0.74 |
| 60V0.75A | 60 | 40 | 0.75 | 1.5 | 0.92 | 3.75 | 5 | 0.25 | 0.42 | 0.62 |
| 60V0.9A | 60 | 40 | 0.9 | 1.8 | 0.99 | 4.5 | 5 | 0.2 | 0.33 | 0.49 |
| 60V1.1A | 60 | 40 | 1.1 | 2.2 | 0.15 | 5.5 | 8 | 0.15 | 0.27 | 0.4 |
| 60V1.35A | 60 | 40 | 1.35 | 2.7 | 0.12 | 6.75 | 8 | 0.12 | 0.21 | 0.32 |
| 60V1.6A | 60 | 40 | 1.6 | 3.2 | 0.09 | 8 | 8 | 0.09 | 0.16 | 0.24 |
| 60V1.85A | 60 | 40 | 1.85 | 3.7 | 0.08 | 9.25 | 8 | 0.08 | 0.14 | 0.21 |
| 60V2A | 60 | 40 | 2 | 4 | 0.07 | 10 | 8 | 0.07 | 0.14 | 0.16 |
| 60V2.5A | 60 | 40 | 2.5 | 5 | 0.05 | 12.5 | 8 | 0.05 | 0.1 | 0.15 |
| 60v3A | 60 | 40 | 3 | 6 | 0.04 | 15 | 8 | 0.04 | 0.08 | 0.12 |
| 60V3.75A | 60 | 40 | 3.75 | 7.5 | 0.03 | 18.75 | 8 | 0.03 | 0.06 | 0.1 |
| 60V5A | 60 | 40 | 5 | 10 | 0.02 | 25 | 8 | 0.02 | 0.06 | 0.1 |

V max  = Maximum operating voltage device can withstand without damage at rated current (Imax).

I max   = Maximum fault current device can withstand without damage at rated voltage (V max).

I hold   = Hold Current. Maximum current device will not trip in 25°C still air.

I trip    = Trip Current. Minimum current at which the device will always trip in 25°C still air.

Pd      = Power dissipation when device is in the tripped state in 25°C still air environment at rated voltage.

Ri min/max  = Minimum/Maximum device resistance prior to tripping at 25°C.

R1max  = Maximum device resistance is measured one hour post reflow.

CAUTION : Operation beyond the specified ratings may result in damage and possible arcing and flame.

Time

TP 260

100

25

Soldering

5Sea.MAX

Temperature(°C)

Gradual

Cooling

Pre-Heating

1 Minute Min.

Soldering Parameters

| WAVE SOLDERING INFORMATIONS |  |
| --- | --- |
| Pre-Heating Zone | Max. ramping rate should not exceed 4℃/Sec. |
| Soldering Zone | Max. solder temperature should not exceed 260℃ |
| Cooling Zone | Cooling by natural convection in air. |
| ◎Specifications are subject to change without notice. |  |

E

C

A

C

B

D

A

D

B

C

Physical Dimensions(mm.)

| Model | A | B | C | D | E | Lead |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max. | Max. | Typ. | Min. | Max. | Style |
| 60v0.05A | 5 | 8.5 | 5.1 | 5 | 3 |  |
| 60v0.1A | 5.5 | 9.5 | 5.1 | 5 | 3 |  |
| 60v0.17A | 7.4 | 12.7 | 5.1 | 5 | 3 |  |
| 60V0.2A | 7.4 | 12.7 | 5.1 | 5 | 3 |  |
| 60V0.25A | 7.4 | 12.7 | 5.1 | 5 | 3 |  |
| 60V0.3A | 7.4 | 13 | 5.1 | 5 | 3 |  |
| 60V0.4A | 7.8 | 13.5 | 5.1 | 5 | 3 |  |
| 60V0.5A | 7.8 | 13.5 | 5.1 | 5 | 3 |  |
| 60V0.65A | 9.7 | 14.5 | 5.1 | 5 | 3 |  |
| 60V0.75A | 10.4 | 15.2 | 5.1 | 5 | 3 |  |
| 60V0.9A | 11.7 | 15.8 | 5.1 | 5 | 3 |  |
| 60V1.1A | 13 | 13 | 5.1 | 5 | 3 |  |
| 60V1.35A | 14.5 | 14.5 | 5.1 | 5 | 3 |  |
| 60V1.6A | 16.3 | 16.3 | 5.1 | 5 | 3 |  |
| 60V1.85A | 17.8 | 17.8 | 5.1 | 5 | 3 |  |
| 60V2A | 17.8 | 17.8 | 5.1 | 5 | 3 |  |
| 60V2.5A | 21.3 | 21.3 | 10.5 | 5 | 3 |  |
| 60v3A | 21.3 | 21.3 | 10.5 | 5 | 3 |  |
| 60V3.75A | 28.5 | 33.5 | 10.5 | 5 | 3 |  |
| 60V5A | 28.5 | 33.5 | 10.5 | 5 | 3 |  |

PHYSICAL SPECIFICATIONS :

Materials : 60V0.05A: Tin-plated nickel-copper alloy, 0.205mm2  (24AWG), Φ0.51mm(0.020 in).

Lead Solderability : MIL-STD-202, Method 208E

Packaging Quantity

| Order information |  |  |  |  |  | Packing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 185 | K or S |  | R or U |  | Model |  | Reel Q'ty | Bag Q'ty |
| Radial type | Hold | K=Kink leads |  | R= Tape & |  |  |  | 3000 | 500 |
| 60 V | Current |  |  | Reel |  |  |  |  |  |
|  | (A) | S=Straight |  | U= Bulk |  |  |  |  |  |
|  |  | leads |  | packaged |  |  |  |  |  |

Tape & Reel packaging per EIA468-B standard.