Performance Specification

| Model |  |  |  |  |  | Maximum Time |  | Resistance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vmax | Imax | Ihold | Itrip | Pd | To Trip |  |  |  |  |
|  |  |  |  |  | Typ. | Current | Time | Rimin | Rimax | R1max |
|  | (Vdc) | (A) | (A) | (A) | (W) | (A) | (Sec) | () | () | () |
| 30V30mA | 30 | 40 | 0.30 | 0.60 | 0.44 | 8.00 | 0.3 | 0.370 | 0.720 | 1.080 |
| 30V40mA | 30 | 40 | 0.40 | 0.80 | 0.45 | 8.00 | 0.3 | 0.250 | 0.430 | 0.645 |
| 30V50mA | 30 | 40 | 0.50 | 1.00 | 0.46 | 8.00 | 0.3 | 0.150 | 0.400 | 0.600 |
| 30V65mA | 30 | 40 | 0.65 | 1.30 | 0.47 | 8.00 | 0.4 | 0.120 | 0.300 | 0.450 |
| 30V75mA | 30 | 40 | 0.75 | 1.50 | 0.48 | 8.0 | 0.4 | 0.100 | 0.250 | 0.375 |
| 30V90mA | 30 | 40 | 0.90 | 1.80 | 0.6 | 4.50 | 5.9 | 0.070 | 0.145 | 0.220 |
| 30V1.1A | 30 | 40 | 1.10 | 2.20 | 0.7 | 5.50 | 6.6 | 0.050 | 0.120 | 0.170 |
| 30V1.35A | 30 | 40 | 1.35 | 2.70 | 0.8 | 6.75 | 7.3 | 0.040 | 0.085 | 0.130 |
| 30V1.6A | 30 | 40 | 1.60 | 3.20 | 0.9 | 8.00 | 8.0 | 0.030 | 0.070 | 0.110 |
| 30V1.85A | 30 | 40 | 1.85 | 3.70 | 1.0 | 9.25 | 8.7 | 0.030 | 0.060 | 0.090 |
| 30V2.5A | 30 | 40 | 2.50 | 5.00 | 1.2 | 12.5 | 10.3 | 0.020 | 0.040 | 0.070 |
| 30V3A | 30 | 40 | 3.00 | 6.00 | 2.0 | 15.0 | 10.8 | 0.020 | 0.050 | 0.080 |
| 30V4A | 30 | 40 | 4.00 | 8.00 | 2.5 | 20.0 | 12.7 | 0.010 | 0.030 | 0.050 |
| 30V5A | 30 | 40 | 5.00 | 10.00 | 3.0 | 25.0 | 14.5 | 0.010 | 0.030 | 0.050 |
| 30V6A | 30 | 40 | 6.00 | 12.00 | 3.5 | 30.0 | 16.0 | 0.005 | 0.020 | 0.040 |
| 30V7A | 30 | 40 | 7.00 | 14.00 | 3.8 | 35.0 | 17.5 | 0.005 | 0.020 | 0.030 |
| 30V8A | 30 | 40 | 8.00 | 16.00 | 4.0 | 40.0 | 18.8 | 0.005 | 0.020 | 0.020 |
| 30V9A | 30 | 40 | 9.00 | 18.00 | 4.2 | 40.0 | 20.0 | 0.005 | 0.010 | 0.020 |

V max  = Maximum operating voltage device can withstand without damage at rated current (Imax).

I max   = Maximum fault current device can withstand without damage at rated voltage (V max).

I hold   = Hold Current. Maximum current device will not trip in 25°C still air.

I trip    = Trip Current. Minimum current at which the device will always trip in 25°C still air.

Pd      = Power dissipation when device is in the tripped state in 25°C still air environment at rated voltage.

Ri min/max  = Minimum/Maximum device resistance prior to tripping at 25°C.

R1max  = Maximum device resistance is measured one hour post reflow.

CAUTION : Operation beyond the specified ratings may result in damage and possible arcing and flame.

Environmental Specifications

| Test |  |  |  | Conditions |  |  |  | Resistance change |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passive aging |  |  |  | +85°C, 1000 hrs. |  |  |  | ±5% typical |  |  |
| Humidity aging |  |  |  | +85°C, 85% R.H. , 168 hours |  |  |  | ±5% typical |  |  |
| Thermal shock |  |  |  | +85°C to -40°C, 20 times |  |  |  | ±33% typical |  |  |
| Resistance to solvent |  |  |  | MIL-STD-202,Method 215 |  |  |  | No change |  |  |
| Vibration |  |  |  | MIL-STD-202,Method 201 |  |  |  | No change |  |  |
| Ambient operating conditions : - 40 °C to +85 °C |  |  |  |  |  |  |  |  |  |  |
| Maximum surface temperature of the device in the tripped state is 125 °C |  |  |  |  |  |  |  |  |  |  |

Time

TP 260

100

25

Soldering

5Sea.MAX

Temperature(°C)

Gradual

Cooling

Pre-Heating

1 Minute Min.

Soldering Parameters

| WAVE SOLDERING INFORMATIONS |  |
| --- | --- |
| Pre-Heating Zone | Max. ramping rate should not exceed 4℃/Sec. |
| Soldering Zone | Max. solder temperature should not exceed 260℃ |
| Cooling Zone | Cooling by natural convection in air. |
| ◎Specifications are subject to change without notice. |  |

R30

135

XXXY

30= Radial type 30 Vrms

135 = 1.35A hold current

XXX= Date code

Y= Factory code

R = Trademark

A

D

B

C

E

C

F

E

A

D

B

C

Physical Dimensions(mm.)

| Model | A | B | C | D | E | F | Lead |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max. | Max. | Typ. | Min. | Max. | Max. | Style |
| 30V30mA | 7.4/0.29 | 10.2/0.4 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V40mA | 7.4/0.29 | 11.4/0.45 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V50mA | 7.4/0.29 | 11.4/0.45 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V65mA | 7.4/0.29 | 11.4/0.45 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V75mA | 7.4/0.29 | 11.4/0.45 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V90mA | 7.4/0.29 | 12.2/0.48 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Kink |
| 30V1.1A | 7.4/0.29 | 14.2/0.56 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Kink |
| 30V1.35A | 8.9/0.35 | 13.5/0.53 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Kink |
| 30V1.6A | 8.9/0.35 | 15.2/0.60 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Kink |
| 30V1.85A | 10.2/0.40 | 15.7/0.62 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Kink |
| 30V2.5A | 11.4/0.45 | 18.3/0.72 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Kink |
| 30V3A | 11.4/0.45 | 17.3/0.68 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V4A | 14.0/0.55 | 20.1/0.79 | 5.1/0.20 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V5A | 14.0/0.55 | 24.9/0.98 | 10.2/0.40 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V6A | 16.5/0.65 | 24.9/0.98 | 10.2/0.40 | 7.6/0.3 | 3.0/0.12 | 1.2/0.05 | Straight |
| 30V7A | 19.1/0.75 | 26.7/1.05 | 10.2/0.40 | 7.6/0.3 | 3.0/0.12 | 2.0/0.08 | Straight |
| 30V8A | 21.6/0.85 | 29.2/1.15 | 10.2/0.40 | 7.6/0.3 | 3.0/0.12 | 2.0/0.08 | Straight |
| 30V9A | 24.1/0.95 | 29.7/1.17 | 10.2/0.40 | 7.6/0.3 | 3.0/0.12 | 2.0/0.08 | Straight |

PHYSICAL SPECIFICATIONS :

Materials : Leads 30V0.3A ~ 2.5A: Tin-platedcopper-cladsteel,0.205mm2(24AWG),Φ0.51mm(0.020in).

30V3A ~ 9A: Tin-plated copper, 0.52mm2  (20AWG), Φ0.81mm(0.032 in).

Lead Solderability : MIL-STD-202, Method 208E

Packaging Quantity

| 30 | 135 | K or S | R or U | Model | Reel Q'ty | Bag Q'ty |
| --- | --- | --- | --- | --- | --- | --- |
| Radial type | Hold | K= Kink leads |  | 30V0.3A ~ 30V0.75A | - | 500 |
| 30 V | Current |  | R=Tape&reel | 30V0.9A ~ 30V2.5A | 3000 | 500 |
|  | (A) | S=Straight | U= Bulk | 30V3A ~ 30V4A | 1500 | 500 |
|  |  | leads | packaged | 30V5A ~ 30V9A | - | 500 |

Tape & Reel packaging per EIA468-B standard.

Cross Reference

| Darong |  |  |  |
| --- | --- | --- | --- |
|  | Tyco / Raychem | Bourns / Multifuse® | Polytronics / EVERFUSE® |
| 30V30mA | - | - | - |
| 30V40mA | - | - | - |
| 30V50mA | - | - | - |
| 30V65mA | - | - | - |
| 30V75mA | - | - | - |
| 30V90mA | RUEF090 | MF-R090-0-009 | RLD30P090UF |
| 30V1.1A | RUEF110 | MF-R110 | RLD30P110UF |
| 30V1.35A | RUEF135 | MF-R135 | RLD30P135UF |
| 30V1.6A | RUEF160 | MF-R160 | RLD30P160UF |
| 30V1.85A | RUEF185 | MF-R185 | RLD30P185UF |
| 30V2.5A | RUEF250 | MF-R250 | RLD30P250UF |
| 30V3A | RUEF300 | MF-R300 | RLD30P300UF |
| 30V4A | RUEF400 | MF-R400 | RLD30P400UF |
| 30V5A | RUEF500 | MF-R500 | RLD30P500UF |
| 30V6A | RUEF600 | MF-R600 | RLD30P600UF |
| 30V7A | RUEF700 | MF-R700 | RLD30P700UF |
| 30V8A | RUEF800 | MF-R800 | RLD30P800UF |
| 30V9A | RUEF900 | MF-R900 | RLD30P900UF |

“Raychem” is a registered trademark of Tyco Electronics.

“Multifuse” is a registered trademark of bourns,Inc.

“EVERFUSE” is a registered trademark of Polytronics Technology Corp.

“SEA-LAND” is a registered trademark of Sea-Land electronic corp.