Performance Specification

| Model |  |  | Vmax | Vmax |  |  | Maximum Time |  | Resistance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ihold | Itrip | Operating | Interrupt | Imax | Pd | To Trip |  |  |  |  |
|  |  |  |  |  | (A) | Typ. | Current | Time | Rimin. | Rimax. | R1max |
|  | (A) | (A) | (Vdc) | (Vrms) |  | (W) | (A) | (Sec) | () | () | () |
| 600V150mA | 0.150 | 0.300 | 60 | 600 | 3.0 | 1.00 | 1.00 | 5.00 | 6.00 | 12.00 | 22.00 |
| 600V160mA | 0.160 | 0.320 | 60 | 600 | 3.0 | 1.00 | 1.00 | 7.00 | 4.00 | 10.00 | 18.00 |

V max  = Maximum operating voltage device can withstand without damage at rated current (Imax).

I max   = Maximum fault current device can withstand without damage at rated voltage (V max).

I hold   = Hold Current. Maximum current device will not trip in 25°C still air.

I trip    = Trip Current. Minimum current at which the device will always trip in 25°C still air.

Pd      = Power dissipation when device is in the tripped state in 25°C still air environment at rated voltage.

Ri min/max  = Minimum/Maximum device resistance prior to tripping at 25°C.

R1max  = Maximum device resistance is measured one hour post reflow.

CAUTION : Operation beyond the specified ratings may result in damage and possible arcing and flame.

Environmental Specifications

| Test |  |  |  | Conditions |  |  |  | Resistance change |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passive aging |  |  |  | +85°C, 1000 hrs. |  |  |  | ±5% typical |  |  |
| Humidity aging |  |  |  | +85°C, 85% R.H. , 168 hours |  |  |  | ±5% typical |  |  |
| Thermal shock |  |  |  | +85°C to -40°C, 20 times |  |  |  | ±33% typical |  |  |
| Resistance to solvent |  |  |  | MIL-STD-202,Method 215 |  |  |  | No change |  |  |
| Vibration |  |  |  | MIL-STD-202,Method 201 |  |  |  | No change |  |  |
| Ambient operating conditions : - 40 °C to +85 °C |  |  |  |  |  |  |  |  |  |  |
| Maximum surface temperature of the device in the tripped state is 125 °C |  |  |  |  |  |  |  |  |  |  |

Ihold Versus Temperature

| Model | Maximum ambient operating temperature (Tmao) vs. hold current (Ihold) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -40°C | -20°C | 0°C | 25°C | 40°C | 50°C | 60°C | 70°C | 85°C |
| 600V150mA | 0.233 | 0.206 | 0.178 | 0.150 | 0.124 | 0.110 | 0.096 | 0.083 | 0.062 |
| 600V160mA | 0.249 | 0.219 | 0.190 | 0.160 | 0.132 | 0.117 | 0.103 | 0.088 | 0.066 |

Time

TP 260

100

25

Soldering

5Sea.MAX

Temperature(°C)

Gradual

Cooling

Pre-Heating

1 Minute Min.

Soldering Parameters

| WAVE SOLDERING INFORMATIONS |  |
| --- | --- |
| Pre-Heating Zone | Max. ramping rate should not exceed 4℃/Sec. |
| Soldering Zone | Max. solder temperature should not exceed 260℃ |
| Cooling Zone | Cooling by natural convection in air. |
| ◎Specifications are subject to change without notice. |  |

Physical Dimensions(mm.)

E

Marking

E

B

D

C

A

E

B

D

| Model | A | B | C | D | E | Lead |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max. | Max. | Typ. | Min. | Max. | Style |
| 600V150mA | 13.5 | 12.6 | 5.1 | 4.7 | 6.0 | Kink |
| 600V160mA | 13.5 | 12.6 | 5.1 | 4.7 | 6.0 | Kink |

PHYSICAL SPECIFICATIONS :

Materials :

600R: Tin-plated copper, 22AWG, Φ0.65mm(0.026 in).

Lead Solderability : MIL-STD-202, Method 208E

Packaging Quantity

| 600R | 150 | RA | B-0.5 | Reel Q'ty |  | Bag Q'ty |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Product | Hold | Rx= | B-x.x= | 600 |  | 500 |  |
| Series | Current | Resistance | Resistance bin range |  |  |  |  |
|  | (mA) | range | within 0.5 ohms |  |  |  |  |
|  |  | (Optional) | in one lot(Optional) |  |  |  |  |

Tape & Reel packaging per EIA468-B standard.